Novel investigation into relative intensity noise and frequency chirp of DFB lasers used as transmitters in optical networks

By

Dr S M Vaezi-Nejad

Senior Lecturer

Faculty of Computing,

London Metropolitan University

North Campus, Holloway Road, London N7 8DB

m.vaezinejad@londonmet.ac.uk

ABSTRACT

Today’s telecomm links typically operate from Mb/s to tens of Gb/s [1-8].  The major technological advantages of optical links has lead to development of all levels of communication from the long haul trunk telephony down to local area networks [3,4]. Although there has been a significant improvement in low noise components used in these systems such as Multi Quantum Well Double Feedback laser devices [5,6] but demand for higher optical data capacity keeps increasing. This lectures focuses on our  investigation into the two very important noise parameters of laser diode Relative Intensity Noise (RIN) and Frequency Excursion (Chirp) [4,7,8]. After a short introduction to optical communication, lasers as transmitters in optical communication networks are briefly described. Our novel techniques for measuring RIN & Chirp together with experimental results and measurement uncertainties of a recently completed research project are then discussed in some detail. In the course of this lecture, it will be explained that RIN is a key parameter for laser components used in telecommunication systems due to the drive to reduce inherent noise, improve overall signal-to-noise ratio and thus increasing achievable communication link length and reduce required component specification. The novel use of a reference noise source in our research project has shown significant advantages, achieving improved sensitivity, reducing measurement accuracy as low as ±1dB and simplifying the system calibration methodology thus improving flexibility.  Laser RIN’s of between 10 to 14dB below the Shot RIN have been measured (typically -170dB/Hz) which is a direct result of the improved system sensitivity.

Our Chirp Measurement system has provided a resolution capability of 100kHz which is proved appropriate and also suitable for the majority of DFB lasers when measuring line width.  Our findings show improved amplitude sensitivity, 7 – 10dB and increased measurement stability due to the use of faraday rotating mirrors.

References

[1]  Parthiban, Rajendran; Leckie, Christopher; Zalesky, Andrew; Zukerman, Moshe; Tucker, Rodney S, 2009, Cost Comparison of Optical Circuit-Switched and Burst-Switched Networks, J. Lightwave Tech, 27(13), pp.2315-2329

[2]  Batayneh, Marwan; Schupke, Dominic A; Hoffmann, Marco; Kirstädter, Andreas; Mukherjee, Biswanath, 2008, Optical Network Design for a Multiline-Rate Carrier-Grade Ethernet Under Transmission-Range Constraints, J. Lightwave Tech,26(1), pp.121-130

[3] Ishida, Osamu; Araki, Soichiro, 2009, Challenging Terabit-Class LAN Over Wide Area Networks, J. Lightwave Tech, 27(12), pp.1947-1956

[4] Park, Soo-Jin; Choi, Young-Bok; Oh, Jung-Mi; Koo, Sang-Geun; Lee, Donghan, 2007, An Evolution Scenario of a Broadband Access Network Using R-SOA-Based WDM-PON Technologies, J. Lightwave Tech, 25(11), pp.3479-3487

[5] Yi Luo; Bing Xiong; Jian Wang; Pengfei Cai; Changzheng Sun; 2006, 40 GHz AlGaInAs Multiple-Quantum-Well Integrated Electroabsorption Modulator/Distributed Feedback Laser Based on Identical Epitaxial Layer Scheme, Jpn. J. Appl. Phys. 45  pp. L1071-L1073

[6]  Ha, W.   Gambin, V.   Wistey, M.   Bank, S.   Yuen, H.   Kim, S.   Harris, J.S., Jr. , 2002  Long wavelength GaInNAsSb/GaNAsSb multiple quantum well lasers, Electronics Letters, 38(6), pp. 277-278 (ISSN: 0013-5194)

[7]  Joindot I., 1992, ‘Measurements of relative intensity noise (RIN) in semiconductor lasers’, J. Phys. III, France 2, pp1591-1603.

[8]  McOmber V., Miller C.M., Stokes L., 1992, ‘Noise and Modulation Distortion Measurements with the Lightwave Signal Analyzer’, HP Lightwave Forum.